The solution to receiving audio distortion in the first 10 seconds on 145MHz and 447MHz when you set the power as middle or low

Please check components marked in red do the modification as per following operation

MD-9600 TOP

Change 104 above 470P into 102(0603) and remove two 104 and one 220R

MD-9600 Bottom

Change 104 into 102(0402) and 473 into 102(0402)